ECGMatcher

User Manual

| Item | Details |
| --- | --- |
| Software Name | ECGMatcher |
| Software Version | v1.0.0 |
| Protected Build | Portable Windows x64 Distribution |
| Supported Platform | 64-bit Windows 10/11 |
| Distribution Format | Portable ECGMatcher.exe Application |
| Document Number | ECGM-UM-CN-1.0.0 |
| Document Date | July 2026 |
| Prepared by | Zhongke Huairou (Beijing) Technology Co., Ltd. |

Critical Runtime Requirements `ECGMatcher.exe` and the `_internal` folder must always remain in the same directory. When using or distributing the software, the entire `ECGMatcher` folder must must be copied, extracted, and retained as a complete package. Do not copy or distribute the executable file alone. The software must be run from a directory with write permissions so that training logs, reconstructed ECG images, and experimental results can be generated within the application directory.

For research, education, algorithm validation, and ECG abnormality-screening support only, it does not constitute a medical diagnosis.

Document Control and Scope of Application

| Item | Description |
| --- | --- |
| Intended Users | Reviewers, researchers, software testing personnel, users from collaborating institutions, and authorized personnel conducting non-commercial evaluations |
| Applicable Software | ECGMatcher v1.0.0 Protected Portable Distribution for Windows x64 |
| Core Functions | Standard-template construction, 12-lead ECG signal reconstruction, curve-oriented semantic segmentation, contour-overlap analysis, key-waveform morphology comparison, similarity assessment, and abnormality localization |
| Data-Processing Mode | Offline processing performed on the local computer; normal operation does not require patient data to be uploaded to an external server |
| Source-Code Status | The distribution does not include the ECGMatcher core source code; core modules are provided as native Windows .pyd extensions |
| Clinical Limitations | All outputs are intended solely for research-oriented screening and auxiliary review and must be interpreted by qualified professionals in conjunction with the original electrocardiogram and relevant clinical information |

About This Manual This manual is intended for end users of the software and does not include proprietary build projects, core algorithm source code, or developer-specific packaging details. This manual is publicly released together with the LICENSE, DISCLAIMER, THIRD_PARTY_NOTICES, CITATION, and other accompanying files.

Contents

1 Software Overview and Research Positioning	1

2 Distribution Package Contents and Deployment Requirements	1

3 System Environment and Operating Requirements	2

4 Initial Launch and Software Verification	3

5 Preparation of Patient Input Data	3

5.1 Standard 12-Lead Order	3

5.2 Supported CSV Structures	3

5.3 Data Preparation Recommendations	3

6 Standard Template Preparation and Reuse Strategy	4

6.1 Standard Image Naming	4

6.2 Template Caching	4

7 Graphical User Interface and Functional Areas	4

8 Complete ECGMatcher Workflow	5

8.1 Recommended Initial Testing Procedure	5

8.2 Description of Processing Stages	5

9 Dataset Identification, Caching, and Repeated Execution	6

9.1 Dataset Name Inference	6

9.2 Reuse of Existing Results	6

10 Results Visualization and Metric Interpretation	7

10.1 Image Panels	7

10.2 Key Metrics	7

10.3 Waveform-Segment Severity Grades	7

10.4 Reconstruction and Segmentation Evaluation Metrics	8

11 Output Directories and File Archiving	8

12 Software Integrity	9

13 Frequently Asked Questions and Troubleshooting	9

14 Data Security, Ethical Boundaries, and Clinical Limitations	10

15 Public GitHub Release and Citation Requirements	10

15.1 Repository Documentation Files	10

15.2 Usage and References	11

1 Software Overview and Research Positioning

ECGMatcher is a software platform designed for morphological matching and abnormality screening of image-based or digital multi-lead electrocardiograms. It integrates standard-template construction, patient ECG signal reconstruction, semantic segmentation of waveform morphology, contour-overlap analysis, morphological comparison of key waveform segments, similarity assessment, and abnormality localization within a unified graphical user interface. The software is intended to provide a portable research tool for reusing historical ECG records, validating algorithms, performing batch-wise case comparisons, and conducting interpretable abnormality screening.

The software primarily processes standard 12-lead ECGs and supports the LUDB and PTB-XL datasets, as well as other datasets that comply with the specified input requirements. The system automatically infers the dataset name from the patient data path and creates dataset-specific directories in the folder containing ECGMatcher.exe for storing signal reconstruction, semantic segmentation, contour comparison, key waveform-segment analysis, and fused abnormality-localization results.

The publicly distributed version adopts a protected native-extension architecture. The graphical user interface and core algorithmic modules are compiled into `.pyd` files using Cython and the Windows C/C++ toolchain, and are subsequently packaged by PyInstaller as a portable Windows onedir application. End users are not required to install Python or access the original algorithm scripts.

Research Use Statement The abnormality scores, abnormal leads, waveform-segment grades, and localized time intervals generated by ECGMatcher represent evidence of morphological deviation relative to the standard template. The software does not directly provide disease diagnoses, etiological determinations, or treatment recommendations and must not be used as the sole basis for emergency management, medication-related decisions, or clinical diagnosis.

2 Distribution Package Contents and Deployment Requirements

ECGMatcher is distributed in the PyInstaller onedir format. The complete public software package is not a standalone ECGMatcher.exe file, but rather an integrated ECGMatcher folder containing the executable, runtime dependencies, reference data, standard templates, and output directories.

| Component | Function | User Requirements |
| --- | --- | --- |
| ECGMatcher.exe | Serves as the graphical user interface and primary workflow entry point | Launch by double-clicking. Do not copy or run it separately from the _internal directory |
| _internal | Contains the Python runtime, PySide6, PyTorch, OpenCV, Matplotlib, core .pyd modules, and other required dependencies | Do not delete, rename, or manually reorganize this directory |
| 10140238.csv | Provides the reference time scale, sampling length, and lead-layout configuration used to construct the standard image-template repository | Must remain in the same directory as ECGMatcher.exe |
| ECG Datasets | Stores patient CSV files. Reconstructed results are written by default to <dataset>_output within the same directory | Separate subdirectories, such as LUDB and PTB-XL, may be created for individual datasets |
| standard img | Contains the 12-lead standard waveform images supplied with the software or selected by the user | In standard-image mode, the complete set of lead images must be retained |
| standard waveform / masks | Caches standard signals, standard reconstruction results, and template masks | Long-term retention is recommended when the template remains unchanged |
| logs | Stores complete UTF-8-encoded backend logs organized by dataset and execution time | Must be reviewed during troubleshooting and experimental archiving |

Recommended Deployment Location The complete software package should be extracted to a short directory path for which the user has write permissions, such as `D:\ECGMatcher` or `E:\ResearchTools\ECGMatcher`. Running the software directly from within the ZIP archive is not recommended, nor should it be installed in a permission-restricted directory such as `C:\Program Files`. For desktop access, create a shortcut to `ECGMatcher.exe` rather than moving or copying the executable itself to the desktop.

3 System Environment and Operating Requirements

| Item | Minimum Requirement | Recommended Configuration |
| --- | --- | --- |
| Operating System | Windows 10/11, 64-bit | Windows 10/11 with the latest stable updates, 64-bit |
| Processor | Multi-core CPU supporting the 64-bit instruction set | Intel Core i7, AMD Ryzen 7, or higher |
| Memory | 8 GB | 16 GB or more; 32 GB is recommended for batch case processing |
| Graphics Processing Unit | CPU-based operation is supported | NVIDIA GPU compatible with the packaged PyTorch environment, with at least 8 GB of video memory |
| Disk Space | Sufficient space for the software package and results from a single case | At least 10 GB of available space; additional capacity should be allocated according to the number of cases processed in large-scale batch analyses |
| Display Environment | Capable of displaying a resolution of 1366 × 768 | Resolution of 1920 × 1080 or higher, with a display scaling factor of 100%-125% |
| Network Connection | No internet connection is required for normal operation | Required only for downloading the software, accessing documentation, or obtaining software updates |

During the initial launch, the software must initialize PySide6, OpenCV, Matplotlib, PyTorch, and the protected core modules. Consequently, startup may take longer than that of a typical lightweight desktop application. Users should allow sufficient time during the first execution or initial antivirus scan and avoid repeatedly double-clicking the executable.

The software can operate in a CPU-only environment; however, semantic segmentation of ECG waveforms, high-resolution image generation, and batch case analysis may require substantially longer processing times. GPU utilization depends on the PyTorch/CUDA configuration included in the distribution package, the graphics driver installed on the target computer, and the model inference settings.

4 Initial Launch and Software Verification

Fully extract `ECGMatcher_Windows_x64_Protected_v1.0.0.zip` obtained from GitHub. Verify that `ECGMatcher.exe`, `_internal`, `10140238.csv`, and the standard template directories are located within the same ECGMatcher root directory. Double-click `ECGMatcher.exe` and proceed only after confirming that the software was obtained from the official project repository. After the interface opens, select Chinese, English, or Russian from the language drop-down menu. The selected language is applied simultaneously to both the user-interface text and the structured runtime logs. Before initiating formal data processing, validate the complete workflow using a single-case CSV file with a clearly defined structure.

5 Preparation of Patient Input Data

The “Patient Input Path” field accepts either a single CSV file or a folder containing CSV files. When a folder is selected, the software recursively identifies all `.csv` files within that folder and displays the total number of input files in the user interface. It is recommended that each CSV file correspond to a single case. The filename stem is used as the case identifier and the name of the corresponding output directory.

5.1 Standard 12-Lead Order

I, II, III, aVR, aVL, aVF, V1, V2, V3, V4, V5, V6

5.2 Supported CSV Structures

| Format | Typical Structure | Description |
| --- | --- | --- |
| Wide-format table with headers | Column names include I, II, III, aVR, aVL, aVF, and V1–V6; an optional time, t, or sec column may be included | Each row represents one sampling time point |
| Wide-format table without headers | 12 or 13 numerical columns | For a 13-column file, the first column may be interpreted as the time column, and the remaining columns are read according to the standard lead order |
| Transposed or row-oriented structure | Lead names appear in the first row or first column, followed by the corresponding sample values | The software identifies the structure using recognized lead aliases and the numerical data arrangement |
| Long-format table | Contains lead or channel and value or signal columns, with an optional time column | The software groups the data by lead and performs standardized reconstruction |

5.3 Data Preparation Recommendations

- Case filenames should be unique and consistent. It is recommended that filenames contain only letters, numbers, underscores, or hyphens. Duplicate CSV filenames across different subdirectories should be avoided. UTF-8 or UTF-8-SIG encoding is preferred; the software also attempts to decode files using GBK, GB18030, and Latin-1.

- Do not place experimental summary tables, performance metric tables, standard-template CSV files, or other non-case CSV files in the patient input directory.

- Missing leads, substantial inconsistencies in sampling length, incorrect time scales, or extensive missing values may result in reconstruction failure or render the results unsuitable for comparison.

- Before initiating formal batch processing, select one representative case and verify the complete workflow, including CSV Reconstruction, Step 1, Step 2, Step 3, and Step 4.

6 Standard Template Preparation and Reuse Strategy

The “Standard Template Path” field supports a folder containing standard template masks, a standard template CSV file or CSV folder, or a folder containing standard 12-lead waveform images. These three input modes differ in computational cost and template reuse strategy.

| Template Type | Selection Method | System Behavior | Recommended Use Case |
| --- | --- | --- | --- |
| Standard template mask folder | Select a folder containing *_trace_mask.png files | Directly reuses the standard masks and skips the Standard Library and Standard Mask stages | Repeated experiments, manuscript result verification, and rapid testing |
| Standard template CSV file or CSV folder | Select a single CSV file or a directory containing CSV files | Reconstructs the standard ECG signals and subsequently generates the standard template masks | Availability of digital standard ECG signals |
| Standard 12-lead waveform images | Select a folder containing 12 individual lead images | Uses 10140238.csv in the root directory to construct the standard signal library, followed by waveform reconstruction and segmentation | Initial construction of the standard library or image-based template sources |

6.1 Standard Image Naming

Supported image formats include PNG, JPG, JPEG, BMP, TIF, TIFF, and WEBP. For the public distribution package, use of the project-provided `standard img` folder is recommended to avoid inconsistencies in lead identification caused by customized filenames.

6.2 Template Caching

The software records a signature for each standard template. When the template remains unchanged and the `standard waveform`, `standard waveform_masks`, and associated manifest files are complete, the existing standard signals, reconstructed images, and masks can be reused. If the template path or template content changes, the system automatically rebuilds the standard template.

Standard Template Consistency For reproducibility studies, the standard template, reference CSV file, model weights, and data preprocessing protocol must remain fixed. Variations in template morphology, sampling scale, or image clarity directly affect contour-overlap analysis, key waveform-segment comparison, and the final abnormality scores.

7 Graphical User Interface and Functional Areas

| Functional Area | Primary Controls/Contents | Functional Description |
| --- | --- | --- |
| Patient Input Area | Patient input path, Select File, Select Folder, Clear, and input count | Specifies a single-case or batch CSV input and displays the number of detected input files |
| Standard Template Area | Standard template path, Select File, Select Folder, and Clear | Specifies a standard mask folder, standard CSV file, or standard image folder |
| Language and Execution Area | Language, Run ECGMatcher, Refresh Results, and User Guide | Switches among Chinese, English, and Russian; initiates the workflow or rescans existing results |
| Case and Lead Selection Area | Current case and current lead | Enables navigation among batch cases and among ECG_reconstructed, leads I–V6, and II_rhythm |
| Six Image Panels | Original input image, semantic segmentation image, difference heatmap, overlap relationship image, deviation fusion image, and key waveform-segment abnormality overlay | Synchronously displays multistage evidence for the currently selected case and lead |
| Key Metrics Area | Fused abnormality score, fused similarity, mean vertical deviation, and most abnormal lead | Presents the principal quantitative results at the case or lead level |
| Waveform-Segment Results Area | Abnormal waveform segments and segment summary | Displays the scores and severity grades for the P wave, QRS complex, T wave, and ST–T segment |
| Text and Progress Area | Clinician-oriented auxiliary recommendations, runtime logs, and stage-specific progress bars | Provides research-oriented review guidance and displays the execution status of each backend processing stage |

“Refresh Results” only rescans the Step 4 case results stored on disk and updates the case list, lead list, and image panels; it does not rerun the algorithms. During processing, do not move the patient CSV files, standard templates, or software root directory.

8 Complete ECGMatcher Workflow

Standard Template Preparation → Patient CSV Reconstruction → Step 1: ECG Waveform Semantic Segmentation → Step 2: Contour-Overlap Analysis → Step 3: Morphological Comparison of Key Waveform Segments → Step 4: Similarity Assessment and Abnormality Localization → Integrated Visualization in the Graphical User Interface

8.1 Recommended Initial Testing Procedure

- Copy one validated 12-lead ECG CSV file to a separate test folder.

- Preferentially select the existing `standard waveform_masks` directory as the standard template to minimize variables during the initial test.

- Launch `ECGMatcher.exe` and select the patient CSV file and standard template path.

- Select the interface language and click “Run ECG Matcher.”

- Confirm that CSV Reconstruction, Step 1, Step 2, Step 3, and Step 4 are completed sequentially.

- Switch among the current case and multiple leads, and inspect the six image panels, key metrics, and abnormal waveform-segment table.

- Review the complete UTF-8-encoded logs stored in `logs/<dataset>/<timestamp>`.

8.2 Description of Processing Stages

| Stage | Primary Task | Success Criterion |
| --- | --- | --- |
| Standard Library | Constructs the standard digital signal library from the 12-lead standard images and 10140238.csv | Generates complete standard CSV files and standard waveform results; this stage may be reused or skipped when standard masks or standard CSV files are provided |
| CSV Reconstruction | Parses patient or standard CSV files and generates individual images for the 12 leads, the II_rhythm image, the complete composite ECG image, foreground masks, and reconstruction evaluation results | Complete case-level reconstructed images and masks are generated; a compatible parsing branch is invoked automatically when necessary |
| Standard Mask | Performs waveform foreground segmentation on the reconstructed standard images | Generates valid *_trace_mask.png files; waveform-structure priors are automatically applied if the initial segmentation fails |
| Step 1 | Performs semantic segmentation of waveform morphology on the reconstructed patient ECG images | Valid lead masks are generated for every case lacking existing segmentation results |
| Step 2 | Aligns and overlays the standard template and patient waveforms, generates difference heatmaps, and computes local deviations | Complete lead-level outputs are generated for each case |
| Step 3 | Performs morphological comparison of the key P-wave, QRS-complex, T-wave, and ST–T segments | Generates abnormal waveform-segment intervals, scores, and overlay images |
| Step 4 | Integrates evidence from Steps 2 and 3 to calculate similarity scores, abnormality scores, and localization results | Generates valid case summaries, final quantitative metrics, and visualizations |

The software performs case-level completeness validation. Even when a processing stage terminates normally, it is still considered unsuccessful if the required case directory or critical output files are missing, thereby preventing erroneous progression to subsequent stages. Step 2 will not be initiated unless Step 1 has been successfully validated.

9 Dataset Identification, Caching, and Repeated Execution

9.1 Dataset Name Inference

The software infers the dataset name from the patient input path. If the path contains `LUDB`, the dataset is identified as LUDB. If it contains `PTB-XL`, `PTBXL`, or `PTB_XL`, the dataset name is standardized as PTB-XL. For other datasets, the nearest valid parent-directory name is used, and characters prohibited in Windows paths are automatically removed or replaced.

9.2 Reuse of Existing Results

- If the selected case already has complete Step 4 results, the software displays the existing results directly without rerunning the full workflow.

- When only some cases in a batch input have existing results, the software processes only the cases with missing results.

- If a previously processed case has complete Step 4 results but lacks the finalized reconstructed Raw Input image, the software repairs only the reconstruction output without recomputing Steps 1–4.

- If the patient CSV content or standard template signature changes, the corresponding cache is invalidated and the affected results are recomputed.

- Historical results with a complete directory structure remain compatible for display even when no execution manifest is available.

- The software completes all processing stages for missing cases within a temporary workspace. Results are merged into the final output directory only after CSV Reconstruction, Step 1, Step 2, Step 3, and Step 4 have all passed case-level validation, thereby reducing the risk that incomplete outputs may overwrite existing experimental results.

10 Results Visualization and Metric Interpretation

10.1 Image Panels

| Panel | Data Source | Primary Interpretation |
| --- | --- | --- |
| Original Input Image | CSV Reconstruction | Displays the finalized reconstructed waveform, including the lead title, Time (s), Amplitude (mV), coordinate-axis ticks, black border, and lead-specific color |
| Semantic Segmentation Image | Step 1 | Assesses the foreground waveform displayed as white traces on a black background, with particular attention to waveform continuity and discontinuities |
| Difference Heatmap | Step 2 | Visualizes local contour deviations of the patient waveform relative to the standard template |
| Overlap Relationship Image | Step 2 | Illustrates the spatial relationships among the standard waveform, patient waveform, and overlapping regions |
| Deviation Fusion Image | Step 4 | Displays local temporal deviations and fused evidence used for abnormality localization |
| Key Waveform-Segment Abnormality Overlay | Step 3 | Displays abnormal regions and time intervals associated with the P wave, QRS complex, T wave, and ST-T segment |

10.2 Key Metrics

| Metric | Definition | General Interpretation |
| --- | --- | --- |
| Fused Abnormality Score | Integrated evidence of abnormality derived from global contour characteristics, key waveform segments, and local temporal deviations | A higher value indicates a more pronounced morphological deviation from the standard template |
| Fused Similarity | Overall degree of similarity between the standard template and the patient waveform | A higher value indicates greater consistency in overall waveform morphology |
| Mean Vertical Deviation | Mean pixel-wise deviation between the centerline of the patient waveform and that of the standard template | A lower value indicates greater similarity in waveform position and amplitude morphology |
| Most Abnormal Lead | The lead with the highest fused abnormality score in the current case | Used to prioritize manual review and should not be interpreted as a disease diagnosis |

10.3 Waveform-Segment Severity Grades

| Grade | Abnormality Score Range | Research-Oriented Interpretation |
| --- | --- | --- |
| Normal / Blank | 0 or no valid score | No definitive evidence of waveform-segment abnormality is identified; interpretation should still consider the other leads and the original ECG |
| Mild | 0 < score < 0.45 | Mild deviation; review of adjacent leads and waveform continuity is recommended |
| Moderate | 0.45 ≤ score < 0.65 | Moderate deviation; the relevant waveform segment and local time interval should be examined carefully |
| Severe | score ≥ 0.65 | Pronounced deviation; manual review should be prioritized |

10.4 Reconstruction and Segmentation Evaluation Metrics

| Task | Metrics | Preferred Direction | Description |
| --- | --- | --- | --- |
| Signal Reconstruction | SSIM, PSNR, and PCC | Higher is better | Measure structural similarity, peak signal-to-noise ratio, and linear correlation, respectively |
| Signal Reconstruction | MAE, RMSE, and DTW | Lower is better | Measure absolute error, root mean square error, and dynamic time warping distance, respectively |
| Semantic Segmentation | IoU, Dice, Precision, Recall, and F1-score | Higher is better | Quantify the overlap, accuracy, and completeness of the waveform foreground region |
| Morphological Matching | Overlap ratio, vertical deviation, waveform-segment abnormality score, and fused similarity | Depends on the metric definition | Characterize global and local morphological differences between the standard template and the patient waveform |

Result Interpretation Principles Results should be interpreted jointly across multiple leads. A high abnormality score in a single lead may reflect a genuine morphological change, but it may also be influenced by baseline drift, an incorrect sampling scale, missing leads, waveform discontinuities, template mismatch, or image-reconstruction errors. Research reports should preserve the input CSV files, template version, model-weight version, complete outputs, and runtime logs.

11 Output Directories and File Archiving

All finalized outputs are written by default to the directory containing `ECGMatcher.exe`. Users must ensure that the software root directory has write permissions. Using LUDB as an example, the principal output directories are listed below.

ECG Datasets/LUDB_output/<case>/ step1_LUDB_output_ECGMatcher_masks/<case>/ step2_LUDB_contour_overlap_analysis/<case>/ step3_LUDB_key_segment_morphology_comparison/<case>/ step4_LUDB_similarity_assessment_abnormality_localization/<case>/ logs/LUDB/<run_timestamp>/

| Directory | Primary Contents | Archiving Recommendation |
| --- | --- | --- |
| ECG Datasets/<dataset>_output | Individual 12-lead images, II_rhythm, ECG_reconstructed, PDF files, foreground masks, and reconstruction evaluation files | Retain this directory because the Raw Input display depends on it |
| step1_<dataset>_output_ECGMatcher_masks | Patient waveform semantic-segmentation masks, probability maps, and associated evaluation results | Retain |
| step2_<dataset>_contour_overlap_analysis | Aligned masks, overlap images, difference heatmaps, sliding-window metrics, and deviation curves | Retain |
| step3_<dataset>_key_segment_morphology_comparison | P-wave, QRS-complex, T-wave, and ST–T segment comparisons, abnormal intervals, overlay images, and summary tables | Retain |
| step4_<dataset>_similarity_assessment_abnormality_localization | Fused metrics, case summaries, abnormality-localization results, and final visualizations | Contains the principal final results and must be archived |
| logs/<dataset>/<timestamp> | Original UTF-8-encoded logs and backend configuration records for each processing stage | Retain for troubleshooting, manuscript verification, and software acceptance testing |

After each formal experiment, it is recommended that the dataset results, logs, and version files within the complete ECGMatcher folder be copied to a date-labeled, read-only archive directory. The archive record should include the software version, standard-template version, model-weight version, and input case list.

12 Software Integrity

The core graphical user interface and algorithmic logic of ECGMatcher in the public distribution package have been compiled as native Windows `.pyd` extensions. Software protection is jointly implemented through technical safeguards, the research-use license, copyright notices, third-party license disclosures, and release-management procedures.

13 Frequently Asked Questions and Troubleshooting

| Symptom/Error | Possible Cause | Recommended Action |
| --- | --- | --- |
| No interface appears after double-clicking the EXE | The _internal directory is incomplete, files were quarantined by antivirus software, initial loading is slow, or the program is being run directly from the ZIP archive | Fully extract the package, verify the _internal directory, allow sufficient initialization time, and review quarantine records in Windows Security |
| Missing-module error after copying only the EXE | ECGMatcher.exe has been separated from the _internal directory | Restore the complete ECGMatcher folder and create only a desktop shortcut to ECGMatcher.exe |
| 10140238.csv / Reference CSV not found | The reference CSV file was deleted, renamed, or is no longer located in the same directory as the EXE | Restore 10140238.csv from the official distribution package |
| CSV Reconstruction failed | The CSV structure cannot be recognized, one or more leads are missing, the file is corrupted, the directory is not writable, or the input path was moved | Verify the 12-lead format, test a single case, inspect the CSV Reconstruction log in logs, and move the software to a writable directory |
| No patient case folders found | Step 1 did not generate valid case masks, or an outdated workflow incorrectly proceeded to Step 2 | Review the Step 1 logs and outputs, use the latest distribution package, and confirm that the input case names match the reconstruction directories |
| Processing is complete, but the current case field is empty | Step 4 did not generate a valid case directory, or an incorrect dataset result directory was selected | Review the Step 4 log, click “Refresh Results,” and inspect the step4_<dataset> directory |
| Image unavailable | The stage-specific image for the selected case or lead is missing, or the historical results contain only a subset of the required panels | Verify the current case and lead, inspect the outputs from each processing stage, and reprocess the affected case if necessary |
| Processing is slow or memory is insufficient | Batch processing, high-DPI images, CPU-based inference, or resource consumption by other applications | Begin with a single case, close training and image-processing software, increase available disk space or memory, and avoid running multiple ECGMatcher instances simultaneously |
| GPU is not used | The graphics driver on the target computer is incompatible with the packaged PyTorch environment, or the software has entered a CPU-based or prior-based processing branch | Review the model and device information in the logs. CPU-based operation remains supported but is slower |
| Results differ from those reported in the manuscript | The model weights, standard template, reference CSV, input preprocessing, or case split differs from the experimental configuration | Verify the weight-loading logs, standard-template version, data protocol, and software version |
| Language or historical path settings are incorrect | ECGMatcher_config.json contains outdated configuration information | Close the software, back up and delete ECGMatcher_config.json from the root directory, and restart the application |

When an error occurs, priority should be given to preserving the text displayed in the error dialog, a screenshot of the current interface, `logs/<dataset>/<timestamp>`, a representative input CSV file, and `ECGMatcher_version.txt`. Any patient identifiers must be de-identified before the materials are shared publicly.

14 Data Security, Ethical Boundaries, and Clinical Limitations

- During normal operation, the software reads CSV files and writes results locally; it does not require patient data to be uploaded to external servers.

- Patient filenames, CSV contents, reconstructed images, abnormal time intervals, and logs may contain sensitive medical information and must be de-identified before public disclosure.

- Users must comply with the requirements of their institutions regarding ethical review, data security, informed consent, and medical information management.

- The software outputs morphology-based screening results relative to a standard template and does not directly determine specific diseases, etiologies, or treatment strategies.

- Clinical interpretation must incorporate the original ECG, patient symptoms, medical history, laboratory findings, and professional physician judgment.

- The software is provided “as is,” and users are responsible for independently validating its applicability to the target data, equipment, and population.

Medical Disclaimer ECGMatcher is not a medical device approved by any regulatory authority. Software outputs must not be used as the sole basis for diagnosis, treatment, emergency care, or medication-related decisions. In any clinically relevant context, all results must be reviewed by appropriately qualified professionals.

15 Public GitHub Release and Citation Requirements

15.1 Repository Documentation Files

| File | Purpose |
| --- | --- |
| README.md | Provides an overview of the software, including a quick start guide, download links, input/output specifications, and a screenshot of the graphical user interface |
| LICENSE.txt | Specifies a custom research and evaluation license or an institutionally approved usage agreement |
| DISCLAIMER.md | Outlines the research-only purpose, clinical boundaries, and liability limitations governing the use of this software |
| THIRD_PARTY_NOTICES.md | Lists the licenses and attributions for third-party dependencies, including PyTorch, PySide6, OpenCV, Matplotlib, NumPy, and SciPy |
| MODEL_CARD.md | Documents the model's intended use, training data, input format, performance benchmarks, limitations, and weight checksums |

15.2 Usage and References

Any use of this software or research findings derived from its outputs must acknowledge the ECGMatcher manuscript and the corresponding GitHub Release version. As the manuscript is currently unpublished, the GitHub repository will be updated accordingly upon its formal publication.

— End of Document —

Zhongke Huairou (Beijing) Technology Co., Ltd · Version 1.0.0 · July 2026